The stratospheric-tropospheric version of TM5

A general overview by J.E.Williams

The purpose of this document is to provide an overview of the recently developed stratospheric-tropospheric version of TM5 (hereafter referred to as ST-TM5). The model is currently being developed and many changes have been made concerning the processes and methods adopted in the code since the original idea to have such a version was first conceived. This document provides some overview regarding the current status in 2007.

General construction of the model:

The TM5 model structure has been developed such that different chemical versions of the model are able to use the same “base” code on which to build. The ST-TM5 version has been coded such that it is now compatible with the most recent base version submitted to the CVS (Aug 2006). Although originally designed to use the base-beta2 version of the TM5 structure for many of the underlying Fortran routines, it has also recently been updated and merged to be part of cycle 3 in the new system meaning that all the most recent bug fixes have been implemented. The specific ST-TM5 files containing the chemistry, photolysis, emissions, heterogeneous formulations, etc. are housed in a subdirectory called ‘st’.  This version of the code has been submitted to the CVS in the projects directory and compiles to a running version*.

Model resolution:

The ST-TM5 model resolution has similar options to those available for the standard TM5 version (Krol et al, 2005). The model contains a Cartesian grid and consists of a two-way nested grid which may zoom over certain regions upto 1º x 1° resolution. It has currently been run on resolutions of both 6ºx4º and 3°x2º with between 25-60 vertical layers. The vertical layers are based on the European Centre for Medium-range Weather Forecasts (ECMWF) vertical resolution of 60 vertical layers up to 0.1hPa. From mid 2006 these fields are available on an updated vertical resolution of 91 layers. For scientific studies, ST-TM5 will be run using between 35-45 model layers, where more resolution is maintained in the UT/LS and lower stratosphere compared to other versions. This definition of the vertical resolution is thought to be optimal for the investigation of process studies in the UT/LS and the stratosphere (Bregman, 2005). More layers are amalgamated when using the 91 layer version of the meteorology. Pre-processing of the meteorological input data is performed in an identical fashion to that conducted for the standard version. There is currently the problem that zooming studies down to 1º x 1° cannot be performed using the second moments advection scheme due to non-convergence issues. However, studies which have compared CH4 distributions in the stratosphere over an Arctic winter suggest that using a 1º x 1° grid resolution with the slopes advection scheme is sufficient for stratospheric studies (Bregman et al, 2006).

Advection and Transport:

As with most other versions of TM5, the ST-TM5 is driven using forecast data from the ECMWF database using a default update interval of 6 hours. The representation of the model winds has recently been updated to ensure mass conservation (Segers et al, 2002) which has made significant improvements to the resulting tracer fields (Bregman et al, 2003). Recently the model has been updated so that it can use various interpolated wind-fields including both 6 and 3 hourly interpolated data (Bregman et al, 2006) and that originating from a Climate Model (the Canadian Middle Atmosphere Model c.f. Bregman ,2006). Advection is performed using the second-order moments scheme of Prather (1986). A reduced polar grid is implemented near the poles to prevent the air mass transport exceeding a grid cell during a single time-step. The consequences of this on tracer transport are discussed in detail in Van den Broek, et al (2003).

Chemical Tracers:

The chemical tracers which are included are entirely characteristic of ST-TM5 and cover the chemical families of Ox, NOx, NOy, ClOx, BrOx, and a subset of the NMHC’s, along with the long-lived greenhouse gases CO, CH4, and N2O. Other species include NO3, SO4, SAD and NAT aerosol (included as 7 size segregated particle bins). The NO3 and SO4 fields are initialized using a standard TM3 input file for 1997 from Dentener (2000). In total, there are 79 individual tracers, with 49 being transported and 30 being non-transported. In order to distribute the higher organics throughout the troposphere and lower stratosphere they are included as transported species. A comprehensive list of both the tracer names and chemical species is provided in Table 1. The Ox, NOx, ClOx and BrOx tracers are the method in which short-lived species such as the Cl and Br radicals are transported. This is necessary due to the relatively long chemical lifetimes of such species high up in the stratosphere and as a way of reducing the number of transported tracers. This approach is a common way of transporting such species in a global CTM which deals with stratospheric chemistry (Chipperfield, 1996). Re-partitioning after transport is done using the percentage composition for a particular grid cell from the previous timestep.

Gas Phase Chemistry:

In total 144 gas phase reactions and 48 photolysis reactions are included in the ST-TM5 chemical mechanism, with a comprehensive list of all reactions being given in Tables 2 and 3, respectively. All reaction data for the chemical rates, absorption characteristics and temperature dependencies have been taken from the latest recommendations (Sander at al, 2006; Atkinson et al, 2004). A look-up table is used for the temperature dependent absorption co-efficients (σ) and quantum yields (φ) for 20 of the chemical species calculated on a resolution of 5ºC. The temperature dependent absorption cross-sections for both NO and O2 (below 202nm) are calculated online using the relevant layer temperatures in conjunction with the parameterizations of Koppers and Murtagh (1996) and Allen and Frederick (1982), respectively. These parameterizations tend to breakdown for high zenith angles (> 85º) in the troposphere meaning that limits are applied within the code as to when the calculations should be performed. The photolysis frequencies (or J values) are calculated online using the modified band approach (Williams et al, 2006) which is active for solar zenith angles up to 92.5º and uses ECMWF data for geo-potential height, pressure, temperature and relative humidity. For the higher peroxy species (e.g. C2H5OOH) the photolysis rates are set equal to that of CH3OOH. Moreover, split product pathways exist for 8 of the chemical species. The tracer field for O3 and the pressure field of each layer (O2) are used to calculate the slant optical depth of the column. The method of applying band limits applies to approximately 25% of the chemical species in order to limit errors introduced by the band approach (see Table 3).  Cloud information is taken from ECMWF (namely the LWC, cloud fraction and IWC), converted to g/cm2 and used in a parameterization which calculates the absorption and scattering contributions to the total optical depth (Slingo, 1989). Ice particles (cirrus) and liquid droplets are treated separately, where the ice particles are assumed to have a hexagonal crystal shape (Stammes at al, 2000). The extent of cloud coverage is accounted for using the random overlap method of Geleyn and Hollingsworth (1979) within the RT solver. Aerosols are prescribed throughout the column using the definitions of Shettle and Fenn (1979), where background aerosol is selected for above the tropopause (predominantly scattering). The code has been constructed so that a more comprehensive description of aerosols can be included. There is currently no link between the tracers SO4_a, NO3_a or SAD to the calculation of photolysis (i.e.) the additional scattering introduced by such fields is currently neglected. The NAT particles can be mainly ignored as it forms predominantly in the Poles during the Polar Winter (darkness). The occurrence of thin tropical NAT clouds could be important although it is likely to be a second-order effect. It is envisaged that more work will be performed so as to improve on the effects of aerosol on actinic flux.

The photolysis module has been tuned for JO2 in order to ensure the formation of O3 in the UT/LS is of a sufficient rate. Actinic fluxes and online J values have been compared for both 25 and 35 levels and give minimal differences at the ground (1-2%) as a result of the different vertical grid.

Heterogeneous Chemistry:

In total 33 heterogeneous reactions are included in ST-TM5. Heterogeneous chemistry occurs on cloud droplets, aerosol, Ice particles (cirrus, sub-visible), NAT and super-cooled ternary solution (STS). These processes can be partitioned between those that are relevant in the troposphere and UT/LS and those relevant exclusively for the Stratosphere.

For the troposphere we inherit the methods used in the tropospheric version of TM5 for the loss of N2O5 on aerosol using the approach of Dentener (1993). The γ value has been updated to 0.02 in line with the global average suggested by Evans and Jacob (2005). The concentration fields of both SO4_a and NO3_a are used to determine the effectiveness of this process (via the active surface area). For the wet scavenging of trace gas species the reader is referred to the section concerning emissions and depositions.

The formation of Ice, NAT and STS is limited to altitudes above the tropopause (i.e.) the online calculation for the presence of such particles is skipped for the troposphere in order to optimize run-times. The formulation for the occurrence of liquid STS is taken from the Mainz stratospheric box model and published in Carslaw et al, 1995. Calculations are only performed for grid cells when T<285K and 1[H2O]<10ppm . The saturation vapour pressure over both Ice and NAT is calculated according the method of Hanson and Mauersberger (1988). The formation of ice can only occur when SVAPICE > 1.0, in combination with a temperature of T< TICE-3. The formation and growth of NAT is calculated according to van den Broek et al (2004), which implements the mathematical approach of Carslaw et al, (2002), and is transported as 7 size segregated tracer species. The formation of NAT may only occur if SVAPNAT > 1.  Gravitational sedimentation of NAT occurs according to their size (thus de-nitrification is taken into account). Reactions on surfaces are calculated according to equation (1):

KHET=γΠr2υCin1+3γr4λ					(1)

Where Ci is the concentration of species i, υ is the mean molecular velocity, r is the mean effective particle radius, λ is the mean free path of a molecule in air (pressure/temperature dependent), γ is the reactive uptake co-efficient and n the particle number density. The reactive uptake co-efficients are taken from Sander, et al (2006). The total number of reactions accounted for on Ice particles, NAT and liquid STS are listed in tables 4-6, respectively.

The current formulation for the presence of Ice seems to under predict the presence of cirrus particles compared to the IWP provided by the ECMWF database. Such data has recently been compared to lidar observations where an excellent correlation was found to exist between the ECMWF IWP and the derived optical depth from the lidar (Immler et al, 2006). However, the quality of such an agreement is latitudinally dependent, where larger differences occur between the values in the tropics compared to the mid-latitudes. Therefore, it is proposed that the IWP will be used in conjunction with the hexagonal crystal shape (one fixed size) to calculate the particle density. The ECMWF temperature data smooths out deviations below Tice, therefore using this as a switch means that the ice particles are rarely activated.

Depositions and Emissions:

Both the emission and deposition terms are calculated in a similar manner to that used in the tropospheric zoom version of TM5 except for the CFC’s and Bromine compounds. Modifications have been made to this individual species involved due to the explicit nature of the chemical scheme in ST-TM5.

The inclusion of anthropogenic source gases such as CFC11 and CFC12 necessitates the implementation of emissions at the ground, where mixing processes transport such species into the stratosphere. For this purpose a polynomial fit has been introduced for five different source gases (namely F11, F12, F113, N2O and CCl4) using empirical fit parameters from the CMDL database (www.cmdl.noaa.gov). The emission of F11 is scaled up 6.3% (for 2000) in order to account for the missing chlorine source of CH3CCl3, where this ratio was derived using the percentage contributions of chlorine sources listed in Engel, et al (2005). For CH3Br, a fixed concentration of 9 pptv is used in line with the suggestions of WMO (2003). The knowledge concerning the emission of bromine compounds is still incomplete therefore we use this pragmatic approach to introducing CH3Br into the system.

For the NMHC’s and greenhouse gases the emission database which is used is either EDGAR 97 or the more recent EDGAR 2000ft., where either the yearly totals are segregated into monthly contributions or individual monthly totals are used.  A main difference concerning the use of emissions compared with other versions of TM5 is that rather than having one large emission routine which contains all the species the code has been broken down such that each type of emission has its own dedicated source code (e.g. CO, CH4, etc.). Moreover, as a consequence of the use of explicit organic tracers, the NMHC emissions need to be assigned to individual species by the application of scaling ratios on the total emission strength from both biomass burning and the anthropogenic component, as proposed by Meijer, 2006. The ratios currently employed are those tested in the MESSy chemistry-climate model from MPI, Mainz. This avoids the construction of separate input files which would otherwise need to be synthesized from the database. One caveat is that for species such as C2H4, which is currently absent from ST-TM5 chemistry, a pseudo-emission of the main product needs to be introduced in order to conserve the total mass of carbon emitted per timestep. Moreover, distribution of such emissions over the first few atmospheric layers is to be included to prevent explosions of (e.g) O3 at the ground level due to the fact that the mixing step comes after the addition of such emissions in the chemical solver.

The deposition is divided into both wet- and dry- deposition terms using the same approach as used in the tropospheric TM5 model. Wet scavenging is included for H2O2, HNO3, CH3OOH, HOCl, HOBr, HCHO, CH3CHO, CH3COOH, CH3OH, HCl and HBr using the Henry’s Coefficients and temperature data taken from Sander (1999). The higher hydrocarbons have limited solubility due the inclusion of insoluble groups such as the ethyl group (CH3CH2). Free radicals such as CH3O2 and HO2 could be added to investigate the global effects of scavenging, where higher peroxy species can be considered to be insoluble. Dry deposition is parameterized according to Ganzeveld and Lelieveld (1996) and included for O3, HNO3, NO, NO2, H2O2, CH3CHO, HCHO, CH3OOH, PAN, CO and HNO4. There are species which maybe included, such as HOBr and HOCl, but the relevant input parameters are needed within the code.

The EBI solver:

Due to the computational restrictions imposed on offline global CTM modeling the efficiency of the chemical solver is of paramount importance to the overall runtime of the model. For this purpose we use the EBI scheme (Hertel, et al, 1993) which has been adopted for use with the modified-CBM4 mechanism in the tropospheric version of TM5. Although the performance of the EBI method has been tested for the old stratospheric reaction mechanism upto 10hPa (van den Broek, et al, 2000) the introduction of new chemical species, reactions and photolysis rates requires that the performance needs to be quantified to provide assurance. Moreover, the extension of the model top introduces a region where a higher concentration of free-radicals exists as a result of more intense radiation. Unfortunately, the FACSIMILIE package used during previous evaluations was not available at KNMI therefore an alternative solver was used for quantifying potential differences. For this purpose, an identical version of the box model using the TWO_STEP iterative solver was produced. Here, a simple explicit Gauss-Seibel technique is used for approximating the implicitly defined BDF formula rather than using a modified Newton method (Verwer, 1994). Tests were then performed on a selection of sample columns taken from TM5 for a 25 level version (upto 0.1hPa). By running the box model for a 5-6 days (without emission or deposition terms) and examining the budgets for the total [Cl] and [Br] in the system as a diagnostic, the efficiency of the EBI solver was investigated. A range of atmospheric layers were selected covering a wide range of initial conditions. The ‘ideal’ solution was derived using a time-step of 10s and subsequent simulations compared with the resulting profiles for all of the chemical species. The two-step solver provides a solution explicitly for every chemical species without the use of the quadratic terms needed in the EBI method for the short-lived free-radical species that are involved in fast non-linear reaction cycles, as well as having an iterative time-step in order to achieve ideal solutions.  For this reason it is rather more computationally expensive than EBI. An advantage is that it has an auto-compilation routine for the production of the chemical terms in the solver meaning that it can be used for finding errors in the manually coded EBI solver.  By performing comparisons of both tracer profiles and the diurnal variation in J values the EBI mechanism was successfully de-bugged and seems to perform well for this suite of chemical species.  Tests were conducted using the time-step selected in the model (1800 s-1), which is further divided in EBI to 300s-1 and trace gas profiles for all species were identical.

Constraints used in the model:

Although the initial philosophy was to build a version of TM5 which is free-running in terms of the total ozone column and associated chemical species, certain constraints are still needed concerning some of the other tracer species for which the knowledge concerning their sources and sinks, chemical lifetime and atmospheric transport are still uncertain.

The tracers SO4_a, NO3_a and SAD are essentially used for the reactive surface area of sulphate and nitrate particles which are suspended throughout the lower atmosphere. These are needed as there are currently no parameterizations included for the formation of such particles in the current version of ST_TM5 (i.e.) no coupling to the trace gas fields. If the model is used for (e.g.) studies concerning the effect of volcanic emissions on the stratosphere such constraints will need to be replaced by an efficient parameterization which links gas phase SO2 to particulate sulphate. In the current implementation these particle tracer fields are nudged after initialization towards the monthly zonal means calculated from the input fields which originate from either satellite (SAGE III) or old model fields (TM3).

For N2O and the CFC’s the use of the CMDL fit with respect to date essentially constrains the fields in the lowest layers equal to the measurements. For CH4 the explicit emissions from (e.g.) EDGAR 2000ft can be replaced by a zonal field for the ground layer which has seasonally dependent mixing ratios from literature sources.

For accurate simulation at the top of the atmosphere it is proposed that the integrated total ozone columns for heights above 0.1hPa as observed by satellite instruments such as MIPAS.

The concentration of CH3Cl is fixed at 0.6 ppbv (Engel et al, 2002) in the lowest 4 layers due to the emission source strengths for this species being unknown. This steady-state concentration is thought to reflect the equilibrium state for this species.

For the bromine species, the Bry (defined as = [BrOx] + [HBr] + [BrCl] + [HOBr] + [BrNO3]) is nudged to 22pptv for the top of the atmosphere according to Salawitch et al., (2005).

Initialization:

To date no good initialization file exists for this version of the model. Annual runs are being performed in order to obtain realistic starting fields for many of the chemical tracer species. For NAT the initialization is set to zero as the occurrence is governed by local temperature histories.

Parallelization:

The code has been tested for upto 9 processors by comparing the total global masses of each respective transported tracer at the beginning and the end of a one hour run. Output was identical which was considered to prove that no MPI bugs exist in the code. The outstanding work relates to the online calculation of the actinic fluxes for which technical personnel at SARA are currently focusing on.

Current status:

Although a comparison of fields with satellite data has yet to be performed the general view of the tracer fields is that the long-lived tracer species look quite acceptable in the main. More effort is now being placed on the more exotic species such a HCl for which there seems to be too much loss. Work is also ongoing to include additional emission files, etc.

References

Allen, M. and J.E.Frederick, Effective photodissociation cross sections for molecular oxygen and nitric oxide in the Schumann-Runge bands, J.Atmos.Sci, 39, 2066-2075, 1982.

Atkinson, R., D.L.Baulch, R.A.Cox, J.N.Crowley, R.F. Hampson Jr., R.G. Hynes, M.E. Jenkin, J.A.Kerr, M.J.Rossi, and J.Troe, Summary of evaluated kinetic and photochemical data for atmospheric chemistry, IUPAC Subcommittee in Gas Kinetic Data Evaluation, Web Version, http://www.iupac-kinetic.ch.cam.ac.uk, 2004.

Bregman, B., A.Segers, M.Krol, E. Meijer, and P van Velthoven, On the use of mass-conserving wind fields in chemistry-transport models, Atms. Chem. Phys., 3, 447-457, 2003.

Bregman, B., personal communication, 2005.

Bregman, B., E.Meijer and R.Scheele, Key aspects of stratospheric tracer modeling, Atms.Chem.Phys, 6, 4529-4543 2006.

Carslaw, K.S., B. Luo and T.Peter, An analytic expression for the composition of aqueous HNO3-H2SO4 stratospheric aerosols including gas phase removal of HNO3, Geophys.Res.Letts, 22(14), 1877-1880, 1995.

Carslaw, K.S., J.A.Kettleborough, M.J.Northway, S.Davies, R.S.Gao, D.W.Fahey, D. Baumgardener, M.P.Chipperfield and A.Kleinbohl, A vortex-scale simulation of the growth and sedimentation of large Nitric Acid Hydrate particles, J.Geophys.Res., 107, doi:10.1029/2001JD000467, 2002.

De Reus, M., H.Fischer, R.Sander, V.Gros, R.Kormann, G.Salisbury, R. van Dingen, J.Williams, M.Zollner and J.Leileveld., Observations and model calculations of trace gas scavenging in a dense Saharan dust plume during MINATROC, Atms.Chem.Phys., 5, 1787-1803, 2005.

Dentener, F.J., Heterogeneous Chemistry in the Troposphere, PhD. Thesis, Univeristy of Utrecht, 1993.

Dentener, F.J., personal communication, 2000.

Engel, A., M.Strunk, M.Müller, H-P Haase, C.Poss, I.Levin and U.Schmidt, Temporal development of total chlorine in the high-latitude stratosphere based on reference distributions of CO2 and SF6, J.Geophys.Res., 107(D12), doi: 10.1029/2001JD000584, 2002.

Evans, M.J. and D.J.Jacob, Impact of laboratory studies of N2O5 hydrolysis on global model budgets of tropospheric nitrogen oxides, ozone and OH, Geophys.Res.Letts., 32, doi : 10.1029/2005GL0222469, 2005.

Geleyn, J.F. and A.Hollingsworth, An Economical analytical method for the computation of the interaction between scattering and line absorption of radiation, Contrib.Atms.Phys., 52(1), 1-16, 1979.

Hertel, O., R.Berkowicz, J.Christensen and O.Hov, test of two numerical schemes for use in atmospheric transport-chemistry models, Atms. Envrion., 27(16), 2591-2611, 1993.

Hanson, D. and G. Mauersberger, Laboratory studies of the nitric acid trihydrate: Implications for the South polar stratosphere, Geophys.Res.Letts, 15, 855-858, 1988.

Immler, F., O.Schrems and K.Kruger, Subvisible cirrus cloud in the upper troposphere: A comparison of lidar observations with ECMWF operational analysis data, EGU abstracts, Vienna, 2006.

Koppers, G.A.A. and D.P.Murtagh, Model studies of the influence of O2 parameterizations in the Schumann-Runge bands on ozone related photolysis in the upper atmosphere, Ann.Geophys., 14, 68-79, 1996.

Krol, M., S.Houwling, B.Bregman, M. van den Broek, A.Segers, P. van Velthoven, W.Peters, F.Dentener, and P.Bergamaschi, TM5: a global two-way nested chemistry transport zoom model: algorithm and applications, Atms.Phys.Chem., 5, 417-432, 2005.

Meijer, E., personal communication, 2006.

Sander, R., Compilation of Henry's Law Constants for Inorganic and Organic Species of Potential Importance in Environmental Chemistry (Version 3),
http://www.mpch-mainz.mpg.de/~sander/res/henry.html

Sander, S.P., R.R.Friedl, A.R.Ravishankara, D.M.Golden, C.E. Kolb, M.J.Kurylo, M.J. Molina, G.K.Moortgart, B.J.Finlayson-Pitts, P.H.Wine, R.E.Huie and V.L. Orkin, , Chemical Kinetics and Photochemical Data for Use in Atmospheric studies, Evaluation No.15, JPL Publication 06-02, 2006.

Salawitch, R.J., D.K.Weisenstein, L.J.Kovalenko, C.E.Sioris, P.O.Wennberg, K.Chance, M.K.W.Ko and C.A.McLinden, Sensitivity of ozone to bromine in the lower stratosphere, Geophys.Res.Letts, 32, doi: 10.1029/2004GL021504, 2005.

Segers,A., P. van Velthoven, B.Bregman and M.Krol, On the computation of mass fluxes for Eulerian transport models from spectral meterological fields, in: Proceedings of the 2002 International Conference on Computational Science, Lecture Notes in Computer Science (LNCS), Springer Verlag, 2002.

Shettle, E.P. and R.W.Fenn, Models for the aerosols of the lower atmosphere and the effects of the humidity variations on their optical properties, Environ.Res.Pap., 676, AFGL-TR-79-0114, 91pp., 1979.

Slingo, A., A GCM parameterization for the shortwave radiative properties of water clouds, J.Atmos.Sci., 46, 1419-1427, 1989.

Stammes, P., R.B.A.Koelemeijer and W.H.Knap, From cloud optical thickness to cloud water path, KNMImemo, 2000.

Van den Broek, M.M.P., A.Bregman and J.Leileveld, Model study of stratospheric chlorine activation and ozone loss during the 1996/1997 winter, J.Geophys.Res., 105, 28961-28977, 2000.

Van den Broek, M., M. van Aalst,  A.Bregman, M.Krol, J.Leileveld, G.Toon, S.Garcelon, G.Hansford, R.Jones and T.Gardiner, The impact of model grid zooming on tracer transport in the 1999/2000 Arctic polar vortex, Atmos.Chem.Phys., 3, 1833-1847, 2003.

Van den Broek, M.M.P., J.E.Williams and A.Bregman, Implementing growth and sedimentation of NAT particles in a global Eulerian model, Atms.Chem.Phys., 4, 1869-1883, 2004.

Verwer, J.G., Gauss-Seidel iteration for stiff ODEs from chemical kinetics, SIAM Journal on Scientifi!Computing 15, 1243 -1250, 1994.

Williams, J.E., J.Landgraf, A.Bregman and H.Walter, A modified band approach for the accurate calculation of on-line photolysis rates in stratospheric-tropospheric  Chemical transport Models, Atms.Chem.Phys., 6, 4137-4161, 2006.

| Name in ST-TM5 | Chemical Species | Comments |
| --- | --- | --- |
| # Transported |  |  |
| Ox |  | Cumulative : O3P + O3 |
| NOx |  | Cumulative : NO + NO2 |
| ClOx |  | Cumulative : Cl + ClO + Cl2O2 |
| BrOx |  | Cumulative : Br + BrO |
| H2O2 | H2O2 |  |
| HNO3 | HNO3 |  |
| HNO4 | HNO4 |  |
| NO3 | NO3 |  |
| N2O | N2O |  |
| N2O5 | N2O5 |  |
| HCl | HCl |  |
| ClNO3 | ClNO3 |  |
| HOCl | HOCl |  |
| OClO | OClO |  |
| HBr | HBr |  |
| BrCl | BrCl |  |
| HOBr | HOBr |  |
| BrNO3 | BrNO3 |  |
| CH4 | CH4 |  |
| CO | CO |  |
| CH2O | HCHO |  |
| CH3O2H | CH3OOH |  |
| Alknit | CH3OONO2 | Transport of NO2 : Important for the stratosphere via PAN |
| F11 | CCl3F |  |
| F12 | CCl2F2 |  |
| F113 | CCl2CClF2 |  |
| CCl4 | CCl4 |  |
| CH3Cl | CH3Cl |  |
| CH3Br | CH3Br |  |
| PAN | CH3C(O)COONO2 |  |
| ACET | CH3C(O)CH3 |  |
| C2H6 | C2H6 |  |
| C3H8 | C3H8 |  |
| MCOOH | CH3COOH |  |
| SO4_a |  | Initialized from TM3 field |
| NO3_a |  | Initialized from TM3 field |
| SAD |  | SAGE II Climatology used for initial field |
| NAT1 | HNO3.3H2O | Formed whenever sufficiently low temperature and pre-cursor gases present |
| NAT2 | HNO3.3H2O | Grows from NAT1 |
| NAT3 | HNO3.3H2O | Grows from NAT2 |
| NAT4 | HNO3.3H2O | Grows from NAT3 |
| NAT5 | HNO3.3H2O | Grows from NAT4 |
| NAT6 | HNO3.3H2O | Grows from NAT5 |
| NAT7 | HNO3.3H2O | Grows from NAT6 |
| CH3OH | CH3OH |  |
| MALD | CH3CHO |  |
| EOOH | CH3CH2OOH |  |
| POOH | CH3CH2CH2OOH |  |
| MGLY | CH3C(O)CHO |  |
| # Non-transported |  |  |
| O3 | O3 |  |
| O3P | O3P |  |
| O1D | O1D |  |
| OH | OH |  |
| HO2 | HO2 |  |
| CH3O2 | CH3O2 |  |
| H2 | H2 |  |
| NO | NO |  |
| NO2 | NO2 |  |
| Cl | Cl |  |
| ClO | ClO |  |
| Cl2O2 | Cl2O2 |  |
| Br | Br |  |
| BrO | BrO |  |
| Hrad | H |  |
| Nrad | N |  |
| EO2 | CH3CH2O2 |  |
| C2O3 | CH3C(O)O2 |  |
| PO2 | CH3CH2CH2O2 |  |
| MGLOH | CH3C(O)CH2O2 |  |
| ACETOOH | CH3C(O)CH2OOH |  |
| ACETOH | CH3C(O)CH2OH |  |
| Passive |  |  |
| NOy | (a) | Diagnostic for strat investigations |
| Cly | (b) | Diagnostic for strat investigations |
| Bry | (c) | Diagnostic for strat investigations |
| ‘air’ |  |  |
| H2O | H2O |  |
| ISOP | ISOP |  |
| O2 | O2 |  |
| CO2 | CO2 |  |

Table 1: The chemical tracers which are included in ST-TM5 with their associated chemical structures. For the diagnostic chemical species the following are defined :

(a) noy = nox + no3 + 2.*n2o5 + hno4 + clno3 + brno3 + pan + nrad + hno3

(b) bry = brox + hbr + hobr + brcl + brno3 + ch3br

(c) cly = clox + ch3cl + clno3 + HCl + hocl+ brcl

| Reaction No. | Nomenclature | # Name In code |
| --- | --- | --- |
| 1 | O(3P) + O3  2O2 | koo3 |
| 2 | O(1D) + H2  OH + H | ko1dh2 |
| 3 | O(1D) + H2O  OH + OH | Ko1dh2o |
| 4 | O(1D) + N2O  NO + NO | Ko1dn2oa |
| 5 | O(1D) + N2O  N2 + O2 | Ko1dn2ob |
| 6 | O(1D) + CH4  OH + CH3O2 | Ko1dch4 |
| 7 | O(3P) + OH  H + O2 | Kooh |
| 8 | O(3P) + HO2  OH + O2 | koho2 |
| 9 | O(3P) + H2O2  OH + HO2 | koh2o2 |
| 10 | H + O3  OH + O2 | kho3 |
| 11 | H + HO2  OH + OH | khho2a |
| 12 | H + HO2  H2 + O2 | khho2b |
| 13 | H + HO2  O(3P) + H2O | khho2c |
| 14 | OH + O3  HO2 + O2 | koho3 |
| 15 | OH + H2  H2O + H | kohh2 |
| 16 | OH + OH  O(3P) + H2O | Kohoh |
| 17 | OH + HO2  H2O + O2 | kohho2 |
| 18 | OH + H2O2  H2O + HO2 | kohh2o2 |
| 19 | HO2 + O3  OH + O2 | kho2o3 |
| 20 | HO2 + HO2  H2O2 + O2 | kho2ho2 |
| 21 | O(3P) + NO2  NO + O2 | kono2 |
| 22 | O(3P) + NO3  NO2 + O2 | kono3 |
| 23 | H + NO2  NO + OH | khno2 |
| 24 | OH + NO3  NO2 + HO2 | kohno3 |
| 25 | N + O3  NO + O2 | ko3n |
| 26 | HNO4 + OH  NO2 + H2O + O2 | kohhno4 |
| 27 | HO2 + NO  NO2 + OH | kho2no |
| 28 | HO2 + NO3  OH + NO2 + O2 | kho2no3 |
| 29 | N + O2  NO + O(3P) | kno2 |
| 30 | N + NO  N2 + O(3P) | Knno |
| 31 | NO + O3  NO2 + O2 | knoo3 |
| 32 | NO3 + NO  NO2 + NO2 | kno3no |
| 33 | NO2 + O3  NO3 + O2 | kno2o3 |
| 34 | OH + CO  H + CO2 | Kohco |
| 35 | CH4 + OH (+ O2)  CH3O2 + H2O | kohch4 |
| 36 | OH + HCHO  CO + HO2 + H2O | kohch2o |
| 37 | CH3OH + OH  HCHO + HO2 | kohch3oh |
| 38 | OH + CH3OOH  CH3O2 + H2O | kohch3o2ha |
| 39 | OH + CH3OOH  HCHO + H2O + OH | kohch3o2hb |
| 40 | HO2 + CH3O2  CH3OOH + O2 | kho2ch3o2 |
| 41 | CH3O2 + CH3O2  HCHO + CH3OH + O2 | k2ch3o2 |
| 42 | CH3O2 + CH3O2  2HCHO + 2HO2 | k2ch3o22 |
| 43 | CH3O2 + NO  HCHO + NO2 + HO2 | kch3o2no |
| 44 | O(3P) + ClO  Cl + O2 | Koclo |
| 45 | O(3P) + OClO  ClO + O2 | Kooclo |
| 46 | O(3P) + HCl  Cl + OH | Kohcl |
| 47 | O(3P) + HOCl  OH + ClO | Kohocl |
| 48 | OH + ClO  Cl + HO2 | Kohcloa |
| 49 | OH + ClO  HCl + O2 | Kohclob |
| 50 | OH + OClO  HOCl + O2 | Kohoclo |
| 51 | OH + HCl  Cl + H2O | Kohhcl |
| 52 | OH + HOCl  ClO + H2O | Kohhocl |
| 53 | HO2 + Cl  HCl + O2 | kho2cla |
| 54 | HO2 + Cl  ClO + OH | kho2clb |
| 55 | HO2 + ClO  HOCl + O2 | kho2cloa |
| 56 | HO2 + ClO  HCl + O3 | kho2clob |
| 57 | NO + OClO  ClO + NO2 | Knooclo |
| 58 | Cl + O3  ClO + O2 | kclo3 |
| 59 | Cl + H2  HCl + H | kclh2 |
| 60 | Cl + H2O2  HO2 + HCl | kclh2o2 |
| 61 | Cl + CH4  HCl + CH3O2 | kclch4 |
| 62 | Cl + HCHO (+ O2)  HCl + CO + HO2 | kclch2o |
| 63 | Cl + CH3OH (+ O2)  HO2 + HCHO + HCl | kclch3oh |
| 64 | Cl + CH3OOH  OH + HCHO + HCl | kclch3o2h |
| 65 | Cl + OClO  2ClO | Kcloclo |
| 66 | Cl + Cl2O2  Cl2 + O2 + Cl | kclcl2o2 |
| 67 | Cl + HOCl  ClO + HCl | Kclhocl |
| 68 | Cl + ClNO3  Cl2 + NO3 | kclclno3 |
| 69 | ClO + NO  Cl + NO2 | Kclono |
| 70 | ClO + CH3O2  HO2 + Cl + HCHO | kcloch3o2 |
| 71 | O(3P) + BrO  Br + O2 | Kobro |
| 72 | O(3P) + HBr  OH + Br | Kohbr |
| 73 | O(3P) + HOBr  OH + BrO | Kohobr |
| 74 | OH + HBr  Br + H2O | Kohhbr |
| 75 | HO2 + Br  HBr + O2 | kho2br |
| 76 | HO2 + BrO  HOBr + O2 | kho2bro |
| 77 | OH + BrO  Br + HO2 | Kohbro |
| 78 | Br + O3  BrO + O2 | kbro3 |
| 79 | Br + HCHO (+ O2)  HBr + CO + HO2 | kbrch2o |
| 80 | Br + OClO  BrO + ClO | Kbroclo |
| 81 | BrO + NO  Br + NO2 | Kbrono |
| 82 | BrO + ClO  Br + OClO | Kbrocloa |
| 83 | BrO + ClO  Br + Cl + O2 | Kbroclob |
| 84 | BrO + ClO  BrCl + O2 | Kbrocloc |
| 85 | BrO + BrO  2Br + O2 | Kbrobro |
| 86 | O(3P) + ClONO2  NO3 + ClO | kclno3o |
| 87 | O(3P) +BrONO2  NO3 + BrO | kbrno3o |
| 88 | O(1D) + CCl3F  3Cl + products | kf11o1d |
| 89 | O(1D) + CCl2F2  2Cl + products | kf12o1d |
| 90 | ClO + ClO  2Cl + O2 | kcloclo1 |
| 91 | ClO + ClO  Cl + OClO | kcloclo2 |
| 92 | O(3P) + HCHO  CO + HO2 + OH | koch2o |
| 93 | OH + CH3Cl  Cl + H2O + products | kohch3cl |
| 94 | OH + CH3Br  Br + H2O + products | kohch3br |
| 95 | O(1D) + CCl2FCClF2  3Cl + products | ko1df113 |
| 96 | O(1D) + CCl4  4Cl + products | ko1dccl4 |
| 97 | Cl + CH3Br  Br + HCl + products | kclch3br |
| 98 | O(1D) + CH3Br  Br + products | ko1dch3br |
| 99 | OH + C2H6  C2H5O2 + H2O | kc2h6oh |
| 100 | C2H5O2 + NO (+ O2)  CH3CHO + NO2 + HO2 | keo2no |
| 101 | C2H5O2 + HO2  C2H5OOH + O2 | keo2ho2 |
| 102 | OH + C2H5OOH  C2H5O2 + H2O | Keoohoh |
| 103 | OH + CH3CHO (+ O2)  CH3CO3 + H2O | Kmaldoh |
| 104 | CH3CHO + NO3 (+ O2)  CH3CO3 + HNO3 | kmaldno3 |
| 105 | CH3CO3 + NO (+ O2)  CH3O2 + CO2 + NO2 | kc2o3no |
| 106 | CH3CO3 + HO2  CH3COOH + products | kc2o3ho2 |
| 107 | CH3CO3 + CH3CO3 (+ O2)  2CH3O2 + 2CO2 | k2c2o3 |
| 108 | OH + PAN  HCHO + NO2 + H2O + CO2 | Kpanoh |
| 109 | OH + C3H8 (+ O2)  C3H7O2 + H2O | kc3h8oh |
| 110 | NO + C3H7O2  NO2 + HO2 + 0.82CH3COCH3 | kpo2no |
| 111 | HO2 + C3H7O2  C3H7OOH + O2 | kpo2ho2 |
| 112 | C3H7OOH + OH  C3H7O2 + H2O | Kpoohoh |
| 113 | CH3COCH3 + OH (+ O2)  CH3COCH2O2 + H2O | Kacetoh |
| 114 | CH3COCH2O2 + NO (+ O2)  CH3COCHO + NO2 + HO2 | Kmglohno |
| 115 | CH3COCHO + OH (+ O2)  CH3CO3 + H2O + CO | Kmglyoh |
| 116 | OH + CH3COOH  CH3O2 + H2O | kmcoohoh |
| 117 | Cl + C2H6 (+ O2)  C2H5O2 + HCl | kc2h6cl |
| 118 | Cl + C3H8 (+ O2)  C3H7O2 + HCl | kc3h8cl |
| 119 | CH3COCH2O2 + CH3O2  0.5 CH3COCHO + 0.5 CH3OH + 0.3 CH3CO3 + 0.8 CH2O + 0.3 HO2 + 0.2 CH3COCH2OH | kmglohmo2 |
| 120 | CH3COCH2O2 + HO2  CH3COCH2O2H | kmglohho2 |
| 121 | CH3COCH2O2H + OH  0.3 CH3COCH2O2 + 0.7 CH3COCHO + 0.7OH | kohaceo2h |
| 122 | CH3COCH2OH + OH  CH3COCHO + HO2 | kohaceoh |
| 123 | NO3 + HCHO  HNO3 + CO + HO2 | kno3ch2o |
| 124 | HNO3 + OH  NO3 + H2O | kohhno3m |
| 125 | O2 + O(3P) + M  O3 + M | koo2m |
| 126 | H + O2 + M  HO2 + M | kho2m |
| 127 | OH + OH + M  H2O2 + M | Kohohm |
| 128 | O(3P) + NO + M  NO2 + M | Konom |
| 129 | OH + NO2 + M  HNO3 + M | kohno2m |
| 130 | NO2 + HO2 + M  HNO4 + M | kho2no2m |
| 131 | NO3 + NO2 + M  N2O5 + M | kno2no3 |
| 132 | CH3O2 + NO2+ M  alknit + M | Kalknitm |
| 133 | ClO + NO2 + M  ClNO3 + M | kclno3m |
| 134 | ClO + ClO + M  Cl2O2 | Kcloclom |
| 135 | BrO + NO2 + M  ClNO3 + M | kbrno3m |
| 136 | CH3CO3 + NO2 + M  PAN + M | Kpanm |
| 137 | HO2 + HO2 + M  H2O2 + M | kho2ho2m |
| 138 | O(3P) + O(3P) + M  O2 + M | Koom |
| 139 | HNO4  NO2 + HO2 | kdhno4 |
| 140 | N2O5  NO3 + NO2 | Kdn2o5 |
| 141 | Alknit  CH3O2 + NO2 | Kdalknit |
| 142 | Cl2O2  ClO + ClO | kdcl2o2 |
| 143 | PAN  CH3CO3 + NO2 | Kdpan |
| 144 | O(1D) + N2  O(3P) + N2 | kauxk1 |
| 145 | O(1D) + O2  O(3P) + O2 | kauxk2 |

## Table 2: A comprehensive list of the gas phase reactions included in ST-TM5 with explicit product formulations. All rate constants and temperature dependencies are taken from the latest recommendations. Temperature dependent reactions are highlighted in blue.

## Photolysis rates

| Reaction no | Nomenclature |  |
| --- | --- | --- |
| 1 | O2 hv O3P + O3P | Jo2 |
| 2 | O3 hv O2 + O1D | Jo3d* |
| 3 | O3 hv O2 + O3P | Jo3p* |
| 4 | NO2 hv NO + O3P | Jno2* |
| 5 | NO3 hv NO + O2 | Jnoo2* |
| 6 | NO3 hv NO2 + O3P | Jno2o* |
| 7 | N2O5 hv NO2 + NO3 | Jno2no3* |
| 8 | N2O hvN2 + O1D | Jn2o* |
| 9 | H2O2 hv 2 OH | Jh2o2* |
| 10 | HNO3 hv OH + NO2 | Jhno3* |
| 11 | HNO4 hv HO2 + NO2 | Jno2ho2a |
| 12 | HNO4 hv OH + NO3 | Jno2ho2b |
| 13 | CH2O hv CO + H + HO2 | Jhcoh* |
| 14 | CH2O hv CO + H2 | Jcoh2* |
| 15 | CH2O hv CO + H+ H | Jhhco* |
| 16 | CH3CHO hv CH4 + CO | Jch4co |
| 17 | CH3CHO hv CH3O2+ CO + HO2 | Jch3o2co |
| 18 | CH3CHO hv CH3CO3 + HO2 | Jc2o3ho2 |
| 19 | CH3COCHO hv CH3CO3 + CO + HO2 | Jch3cocho |
| 20 | CH3COCHO hv 0.5CH4+ 1.5CO + 0.5CH3CHO | Jmglych4 |
| 21 | CH3COCH3 hv CH3CO3 + CH3O2 | Jch3coch3* |
| 22 | CH3COCH3 hv 2CH3O2 + CO | Jacetco* |
| 23 | CH3OOH hv CH3O + OH | Jch3ooh |
| 24 | CH3O2NO2 hv CH3O2 + NO2 | Jch3ono2 |
| 25 | PAN hv CH3CO3 + NO2 | Jpan* |
| 26 | OClO hv ClO + O3P | Joclo |
| 27 | ClNO3 hv Cl + NO3 | Jclno3a* |
| 28 | ClNO3 hv ClO + NO2 | Jclno3b* |
| 29 | CH3Cl hv CH3O2 + Cl | Jch3cl* |
| 30 | CCl4 hv Products + 4Cl | Jccl4* |
| 31 | CF2ClCFCl2 hv Products + 3Cl | Jcfc113* |
| 32 | CCl3F hv Products + 3Cl | Jcfc11* |
| 33 | CCl2F2 hvProducts + 2Cl | Jcfc12* |
| 34 | BrNO3 hv Br + NO3 | Jbrno3* |
| 35 | CH3Br hv CH3O2 + Br | Jch3br* |
| 36 | BrCl hv Br + Cl | Jbrcl* |
| 37 | HCl hv H + Cl | Jhcl |
| 38 | HOCl hv OH + Cl | Jhocl |
| 39 | HOBr hv OH + Br | Jhobr |
| 40 | BrO hv Br + O3P | Jbro* |
| 41 | Cl2O2 hv 2Cl + O2 | Jcl2o2 |
| 42 | H2O hv H + OH | Jh2o |
| 43 | NO hv N + O3P | Jno |
| 44 | CH3COCH2O2HH hv CH3CO3 + HO2 + OH | Jacetooh |
| 45 | CH3COCH2OH hv CH3CO3 + HCHO + HO2 | Jacetoh |
| 46 | C2H5OOHhv CH3CHO + HO2 + OH | = Jch3ooh |
| 47 | C3H7OOHhv 0.82CH3COOH + HO2 + OH | = Jch3ooh |
| 48 | CH3COOHhv CH3O2 + OH | = Jch3ooh |

Table 3: A comprehensive list of all photolysis rates included in ST-TM5. Most rates are calculated explicitly except for the higher peroxy species, and Jch3cooh. Moreover,  Jacetooh and Jacetoh are set scaled using the values given in de Reus, et al (2005). Split pathways exist for 5 of the species. Rates for which band limits are applied are given in red. Chemical species which have temperature dependent absorption parameters are denoted with (*). For details regarding the values adopted the reader is referred to Williams et al, 2006.

Heterogeneous reactions on Ice/cloud :

| Reaction no | Nomenclature | Name In code |
| --- | --- | --- |
| 1 | N2O5 + H2O  2HNO3 | Rn2o5h2o_i |
| 2 | N2O5 + HCl  Cl + NO2 + HNO3 | Rn2o5hcl_i |
| 3 | ClNO3 + H2O  HOCl + HNO3 | Rhclno3h2o_i |
| 4 | HCl + HOBr  BrCl + H2O | Rhclhobr_i |
| 5 | HBr + HOBr  Br2 + H2O | Rhbrhobr_i |
| 6 | BrNO3 + HCl  BrCl + H2O | Rbrno3hcl_i |
| 7 | HBr + ClNO3  HNO3 + BrCl | Rhbrclno3_i |
| 8 | HBr + HOCl  BrCl + H2O | Rhbrhocl_i |
| 9 | BrNO3 + H2O  BrOH + HNO3 | Rbrno3h2o_i |
| 10 | HCl + HOCl  Cl2 + H2O | Rhclhocl_i |
| 11 | ClNO3 + HCl  Cl2 + HNO3 | Rclno3hcl_i |
| 12 | N2O5 + HBr  2Br + NO2 + HNO3 | Rn2o5hbr_i |

## Table 4: Heterogeneous reactions on ice particles included in ST-TM5. All uptake co-efficients are taken from Sander et al, 2003.

Heterogeneous reactions on NAT

| Reaction no | Nomenclature | Name in code |
| --- | --- | --- |
| 1 | N2O5 + H2O  2HNO3 | Rn2o5h2o_n |
| 2 | N2O5 + HCl  Cl + NO2 + HNO3 | Rn2o5hcl_n |
| 3 | ClNO3 + H2O  HOCl + HNO3 | Rhclno3h2o_n |
| 4 | HCl + HOBr  BrCl + H2O | Rhclhobr_n |
| 5 | HBr + HOBr  Br2 + H2O | Rhbrhobr_n |
| 6 | BrNO3 + HCl  BrCl + H2O | Rbrno3hcl_n |
| 7 | HBr + ClNO3  HNO3 + BrCl | Rhbrclno3_n |
| 8 | HBr + HOCl  BrCl + H2O | Rhbrhocl_n |
| 9 | BrNO3 + H2O  BrOH + HNO3 | Rbrno3h2o_n |
| 10 | HCl + HOCl  Cl2 + H2O | Rhclhocl_n |
| 11 | ClNO3 + HCl  Cl2 + HNO3 | Rclno3hcl_n |
| 12 | N2O5 + HBr  2Br + NO2 + HNO3 | Rn2o5hbr_n |

## Table 5: Heterogeneous reactions on NAT particles included in ST-TM5. All uptake co-efficients are taken from Sander et al, 2003.

## Heterogeneous reactions on aerosol/clouds

| Reaction no | Nomenclature | Name in code |
| --- | --- | --- |
| 1 | N2O5 + H2O  2HNO3 | Rn2o5h2o_aq |
| 2 | ClNO3 + H2O  HOCl + HNO3 | Rclno3h2o_aq |
| 3 | ClNO3 + HCl  Cl2 + HNO3 | Rclno3hcl_aq |
| 4 | HCl + HOCl  Cl2 + H2O | Rhclhocl_aq |
| 5 | BrNO3 + H2O  BrOH + HNO3 | Rbrno3h2o_aq |
| 6 | HCl + HOBr  BrCl + H2O | Rhclhobr_aq |
| 7 | HBr + HOCl  BrCl + H2O | Rhbrhocl_aq |
| 8 | HBr + HOBr  Br2 + H2O | Rhbrhobr_aq |

## Table 6: Heterogeneous reactions on liquid STS droplets included in ST-TM5. All uptake co-efficients are taken from Sander et al, 2003.

* Valid as long as the base in cy3 does not undergo a significant change in structure

This is simply to initialize the fields. This can be replaced by a satellite time series if available.